Appendix 1

Form № 24

PROJECT

Investigation of Prompt Fission Neutron Emission in Fission (“ЕNGRIN”)

Theme: “Studies of the interaction of neutrons with nuclei and properties of neutron”

Theme code: 03-4-1128-2017/2022

Authors:

Zeynalov Sh.S - Dubna, JINR

Mitsyna L.V. - Dubna, JINR

Sidorova O.V. - Dubna, JINR

Suchovoy A.M. - Dubna, JINR

Kuznetsov A.N. - Dubna, JINR

Lebedev A.M. - Dubna, JINR

Grigorian R.A - Dubna, Dubna State University

Kamyshnikov D. Y. - Dubna, Dubna State University

Yovanchevich Nicola -Novi-Sad, Serbia. University of Novi-Sad

Semkova Valentina – Sofia, Bulgarian Academy of Science

Project leader: Zeynalov Sh. S.

Project leader deputy: Mytsina L. V.

DATE OF SUBMISSION OF PROPOSAL TO SOD ________________________

DATE OF LABORATORY STC_23.04.2021_DOCUMENT NUMBER ________

DATE OF FIRST APPROVAL OF THE PROJECT________________________

Human resource assessment

Table 1.

Employees involved in the project.

| № | Name of Division / Department / Group | Numberofpersons | Amountofemployment |
| --- | --- | --- | --- |
| 1 | FLNP DNP | Zeynalov Sh.S | full-time |
| 2 | FLNP DNP | Mitsyna L.V | full-time |
| 3 | FLNP DNP | Sidorova O.V | full-time |
| 4 | FLNP DNP | Suhovoy A.V. | full-time |
| 5 | State University Dubna | Grigorian R.A. | part-time |
| 6 | State University Dubna | Kamyshnikov D.Y. | part-time |
| 7 | University Novi-Sad, Serbia | Yovanchevich N. | part-time (by request) |
| 8 | INRNE Sofia, Bulgaria | Semkova V. | part-time (by request) |
| 9 | FLNP Workshop | Kuznetsov A.N. | part-time (by request) |
| 10 | FLNP DNP | Lebedev A.M. | part-time (by request) |

Appendix 2

Form № 25

FORM № 25 PROJECT ENDORSEMENT LIST

Investigation of Prompt Neutron Emission in Fission (“ЕNGREN”)

THEME: « Studies of the interaction of neutrons with nuclei and properties of the neutron»

THEME CODE: 03-4-1128-2017/2022

SIGNATURE 	 			DATE

APPROVED BY THE JINR DIRECTOR 				«____»_______2021

ENDORSED BY

JINR VICE-DIRECTOR							«____»_______2021

CHIEF SCIENTIFIC SECRETARY				 	«____»_______2021

CHIEF ENGINEER							 	«____»_______2021

HEAD OF THE SOD						 	«____»_______2021

FLNP DIRECTOR 						 		«____»_______2021

FLNP CHIEF ENGINEER					 		«____»_______2021

PROJECT LEADER	 					 	«____»_______2021

ENDORSED

PAC for CMP 								«____»_______2021

Abstract

Investigations of prompt fission neutron emission are of importance in understanding the fission process in general and the sharing of excitation energy among the fission fragments in particular. Experimental activities at JINR on prompt fission neutron (PFN) emission are underway for more than 20 year. Main focus lies on investigations of prompt neutron emission from the reactions 252Cf (sf) and 235U(n,f) [2-20,23-26] in the region of the resolved resonances. For the last reaction strong fluctuations of fission fragment mass and the mean total kinetic energy distributions have been observed as a function of incident neutron energy [16, 37]. In addition fluctuations of prompt neutron multiplicities were also observed [44]. The goal of the present study is to verify the current knowledge of prompt neutron multiplicity fluctuations and to study correlations with fission fragment properties. Recent measurement of PFN multiplicity in resonance neutron induced fission of 235U(n,f) reaction [27] reveal surprising result, stimulated us to investigate the PFN multiplicity at IREN with new high efficiency  experimental setup.

Contents:

- Introduction										6

- Current status of PFN investigation						7

- Data analysis procedure								9

- Conclusions									9

1.Introduction

The possible correlation between the variation of prompt fission neutron (PFN) multiplicity and the fission fragments (FF) total kinetic energy (TKE) variation measurement is the goal of the project. Investigations of PFN properties for more than six decades have reached considerable success due to efforts have been applied to modification of method with low geometric efficiency method (LGE) first suggested by H.R. Bowman et al in Phys. Rev. 126, 2120 (1962). C. Budtz-Jorgensen and H.H. Knitter, suggested twin back-to-back ionization chamber (TBIC) for correlated FF and PFN properties investigation. TBIC along with PFN detector located at ~0.5-0.7 m distance along the axis of the chamber was called method of PFN investigation with low geometric efficiency (LGE ~0.001) [1]. This method provides information on main parameters characterizing correlated FF: TKE, masses, PFN multiplicity, and PFN velocity measured by TOF method. Several experiments improved understanding of fission process with a new data obtained on PFN properties in reactions 252Cf(sf), 235U(nth,f) и 235U(nres,f) published in [2-20,23-26]. Interpretation of the experiments was done in the framework of multi modal random neck rupture (MM-RNR) model of low excitation energy fission. The model considers nuclei leaving the compound state by various paths to disintegration. These paths (fission modes) related to Bohr fission channels chosen in stochastic manner by the fissile system. Furman and J. Kliman found link between fission channels and fission modes, providing the way of evaluation the probabilities of fission mode realization using experimental data.

Recently correction of PFN multiplicity dependence on FF mass and TKE for 235U(nth,f)  reaction was reported in [23] (see Fig. 1). Investigations carried out in IBR30 in 1999-2000 pulsed reactor in Dubna [16,17], confirmed existence of TKE variations in neutron resonances first reported by F.-J. Hambsch et al in IRMM [36].  PFN multiplicity measurement with LGE was carried out at GELINA in 2007-2008, using PFN detectors from DEMON collaboration and TBIC loaded by1 mg U235. However, statistical accuracy of data taken in resonances was not enough for reliable PFN multiplicity analysis. Therefore, we developed setup with chamber loaded by 230 mg 235U target (99.999%) and PFN detector composed of 32 fast neutron detection modules (76 mm diameter, 51 mm thickness) located at distance ~54 cm from the target (LEM =0.012). We expect improve the statistical accuracy of measurement at 9.2 flight path of IREN facility (full neutron beam intensity ~2*1011 sec/4π) at least by order of magnitude.

2.Current Status of PFN Investigations

First investigation of PFN multiplicity in resonance induced fission reveal multiplicity variations between 235U(n,f) reaction in resonance energy neutron region [11]. Later in experiments performed in EC-JRC-IRMM [36] variations of TKE in resonance region of neutron energy was observed as demonstrated in Fig. 1. Recently experiments, intended to investigate the correlations

Fig. 1 TKE variations in 235U(nres,f) reaction measured in GELINA (left graph) and IBR30 (right graph)

between FF TKE and PFN multiplicity variations was reported in [27, 33]. Authors used position-sensitive TBIC as FF detector and the array of 12 PFN detection modules. The aim of the experiment was simultaneous measurement of FF TKE and PFN

Fig. 2 Correction of PFN multiplicity dependence on mass&TKE measured in 235U(nth,f) in Dubna. Blue line taken from Ref. [40]. Black line is corrected NuBar(A) improves a mass resolution of FF obtained using 2E method.

multiplicity variation using LEM. However, simultaneous measurement of FF TKE and PFN has limitation on the target material weigh (should be as thin as possible). For PFN multiplicity variation investigations in JINR at IREN facility we divided experiment by two steps. At the first step PFN multiplicity measured with thick target of 230 mg. At this step we expect most possible PFN production/detection efficiency with available neutron beam intensity. During preparation of the

Fig. 3. Experimental setup developed for use in ENGREN project

project test measurements were carried out. Fig. 4 demonstrates simplified setup used to estimate the time required for PFN variation measurement at IREN facility, having beam intensity ~2*1011 n/sec*4π. Distance between fission chamber and the neutron detector was 17.5 cm in order the PFN detecting efficiency to be close to the PFN detection efficiency of the setup shown in Fig. 3. Fig. 4 demonstrates data acquisition system setup, developed for digitization of PFN and fission detector pulses. Data acquisition software was partially ready, but can be used for test experiment. Only two out of 40 digitization channels were used in test experiment. Resonance neutron time of flight (tof) was measured using time stamps provided by CAEN N6730 waveform digitizer. To obtain correct tof value “T-Zero” pulse from the IREN generated just before neutron burst was used to reset the time stamps counter.

Fig. 4 Diagram of the electronic apparatus of data acquisition system

Fig. 5 Simplified setup

Pulse from fission chamber, connected to fission detection initiates time stamp write operation  into waveforms memory buffer. Each fission chamber pulse caused recording of event, consisted of time stamp and two waveforms, recorded from fission chamber and PFN detector.  Data analysis performed offline, creating two types of tof-distributions. In first one each fission event was counted forming tof-distribution. In second one only events where fission events coincided

Fig. 6 Tof-distributions with demand of coincidence between fission chamber (red curves) and without demand of coincidence (blue curves). Graphs plotted for data collected in two days (~45 hours). Right graph demonstrates thermal neutron energy range of tof-distribution)

Fig. 7 Fission chamber and PFN detector coincidence waveforms (left graph). Neutron gamma separation graph (central graph). PFN separated from gammas (right graph demonstrates that share of PFN with demand of coincidence is only ~0.13 part of full FF counts, meaning that PFN share is 0.015 of FF counts

with PFN detector pulse within 80 ns formed conditional tof-distribution as demonstrated on Fig. 6.  Coincidence between fission event and PFN registration can be caused by prompt gammas.  Hence the coincidence events should be refined from gammas using pulse shape analysis as demonstrated in Fig. 6. The share of PFN is only 0.13 of data collected with coincidence between fission chamber and PFN detector and it is only 0.016 of FF counts. From data analysis of measurement with simplified setup we concluded that for strongest resonances 45 hour measurement of PFN multiplicity provides ~3% of statistical accuracy.

3.Data analysis procedure

Data analysis procedure was performed off-line as was described above. Data collected with demand of coincidence with PFN detector first refined from prompt gammas by pulse shape analysis. Then using thermal part of tof-distribution one can obtain average PFN detecting efficiency ξ by normalizing the total number of refined PFN counts to total number of FF. Assuming validity of the formula:

,

Where we use the following definitions: PFN and FF – are the numbers of PFN and FF respectively, detected in the thermal part of tof-distribution. ξ– is the PFN detection efficiency and 2.41 – is the NuBar value for 235U(nth,f) reaction known from the literature. Then for resonances and resonance groups we can use the following formula to calculate the  dependence on neutron energy:

.

4.Conclusions

The share of PFN is only 0.13 of data collected with coincidence between fission chamber, henсe the share of PFN counts is only 0.016 of FF counts. From analysis of data measured with simplified setup (target of 230 mg) we concluded that for strongest resonances 45 hour measurement of PFN multiplicity would give ~3% of statistical accuracy. For statistical accuracy of ~3% in mass&TKE variation with 1 mg target we will need at least 50 weeks of measurement.  Additionally 50 weeks would be necessary for development apparatus and data acquisition software and preparations at the channel 2 of IREN.

Appendix 3

Form № 26

Proposed schedule and necessary resources for implementation of the project

“Investigation of Prompt Neutron Emission in Fission (ЕNGREN)”

| Names of costs, resources, and sources of funding |  |  | Cost (thousands of dollars). установки. Resource requirements. |
| --- | --- | --- | --- |
|  |  |  |  | 2022 | 2023 |
| Main equipment and components | Development and fabrication of the neutron detector shielding |  | 17 | 17 | - |
|  | Development and fabrication of conventional fission ionization chamber |  | 22 | 22 | - |
|  | Fabrication of position sensitive ionization chamber with 128 independent spectroscopic channels |  | 34 | 34 | - |
|  | Development and fabrication of software for data acquisition and analysis |  | 30 | 10 | 20 |
|  | Targets expenditure |  | 10 | 6 | 4 |
|  | Construction and refurbishment works in IREN channel 2 |  | 10 | 4 | 6 |
| Necessary resources | Budget expenditures | Construction engineering | - | - | - |
|  |  | Workshop | 8 | 4 | 4 |
|  |  | IREN (hour) | 20 | 10 | 10 |
|  |  | Theme 1128 | 151 | 107 | 44 |

PROJECT LEADER					Sh.S. Zeynalov

Appendix 4

# Form № 29

Estimated cost of the project “Investigations of Prompt Neutron Emission in Fission (ЕNGREN)”

| № | Description of the cost items | Total cost тыс. $ | 2022 | 2023 |  |
| --- | --- | --- | --- | --- | --- |
| Direct expenses |  |  |  |  |
| 1 | Components | 35 | 10 | 25 |  |
| 2 | Equipment | 34 | 34 | - |  |
| 3 | Payments for research performed under contracts | 40 | 10 | 30 |  |
| 4 | Travel expenses | 14 | 7 | 7 |  |
|  |  | 20 | 10 | 10 |  |
|  |  | 8 | 4 | 4 |  |
| Total |  | 151 | 75 | 76 |  |

PROJECT LEADER __________________ Sh.S. Zeynalov «____» ___________________2021 г. LABORATORY DIRECTOR __________________V.N. Shvetsov «____» ___________________2021 г.

LEADING ENGINEER-ECONOMIST _______________L.S. Ovsyanikova «____» ___________________2021 г.

References

- Nifenecker, H. Prompt neutron yields of the fission fragments of 252Cf as a function of the charge of the fragments / H. Nifenecker, M. Ribrag, J. Frehaut, J. Gauriau // Nuclear Physics A - 1969. - Vol. 131, No. 2. - P. 261-266.

- К.А. Петржак, Г.Н. Флеров, Спонтанное деление урана, Доклады АН СССР - 1940. - Т. 28, № 6. - С. 500-501.

- Hahn O. and F. Strassmann, Nachweis der Entstehung aktiver Bariumisotope aus Uran und Thorium durch Neutronenberstrahlung,  Naturwissenschaften – 1939. -  Vol. 27. -P. 89-95

- L. Meitner and O.R. Frisch, Disintegration of uranium by neutrons: a new type of nuclear reaction, Nature - 1939. - Vol. 143. - P. 239-240.

- O. R. Frisch, Physical evidence for the division of heavy nuclei under neutron

bombardment Nature, 1939. - Vol. 143. - P. 276.

- H. R. Bowman, S. G. Thompson, J. C. D. Milton, W. J. Swiatecki, Velocity and angular distributions of prompt neutrons fromspontaneous fission of 252Cf , Physical Review - 1962. - Vol. 126, No. 6. - P. 2120-2136.

- H. R. Bowman, S. G. Thompson, J. C. D. Milton, W. J. Swiatecki, Further Studies of the Prompt Neutrons from the Spontaneous Fission of Cf252, Physical Review - 1963. - Vol. 129, No. 5. - P. 2133-2147.

- K. Skarsvag, I. Singstad, Angular correlation of fission fragments and prompt gamma rays from spontaneous fission of Cf252,  Nuclear Physics - 1965. - Vol. 62, No. 1. - P. 103-112.

- Sokol E.A., Ter-Akopian G.M, Šaro S., Zeinalov Sh.S., Ivanov M.P., Flerov G.N., On the experimental possibilities of detecting multiple neutrons in massive samples, Nuclear Instruments and Methods in Physics Research Section B: Beam Interactions with Materials and Atoms– 1986. – Vol. 17, No 6. – P.  460-461

- A.Oed, P. Geltenbort, F. Gonnenwein, T. Manning, D. Souque, High resolution axial ionization chamber for fission products Nuclear Instruments and Methods in Physics Research - 1983. - Vol. 205, No. 3. - P. 615 – 617

- C. Budtz-Jorgensen and H.-H. Knitter, Simultaneous investigation of fission fragments and neutrons in 252Cf(sf), Nuclear Phys A – 1988. - Vol. 490, P. 307 – 328.

- A.S. Vorobyev, O.A. Sherbakov, Yu.S. Pleva, A.M. Gagarski, G.V.Valski, G.A.Petrov, V.I.Petrova, T.A. Zavarukhina, Measurements of angular and energy distributions of prompt neutrons from thermal neutron-induced fission  Nuclear Instruments and Methods in Physics Research A - 2009. - Vol. 598. - P. 795-801.

- S.S. Kapoor, R. Ramanna, P.N. Rama Rao, Emission of Prompt Neutrons in the Thermal Neutron Fission of U235  Physical Review – 1963. –Vol. 131, -P. 283-296.

- M.S. Samant, R.P. Anand, R.K. Choudhury M. S. Samant, S. S. Kapoor, and D. M. Nadkarni, Prescission neutron emission in 235U(nth,f) through fragment-neutron angular correlation studies Physical Review C -1995. –Vol.  51. –P. 3127-3135.

- S. Oberstedt, F.-J. Hambsch, and F. Vives, Fission-mode calculations for 239U, a revision of the multi-modal random neck-rupture model, Nuclear Physics A – 1998. – Vol. 64. No. 4. – P. 289-305.

- Zeinalov Sh.S., Florek M., Furman W.I., Kriatchkov V.A., Zamyatnin Yu. S., Neutron energy dependence of 235U(n,f) mass and TKE distributions around 8.77 eV resonance VII International Seminar on Interaction of Neutrons with Nuclear – Dubna: JINR, -1999. -E3-1999-212. –P. 258-262.

- Zeinalov Sh.S., Florek M., Furman W.I., Kriachkov V.A., Zamyatnin Yu.S, Neutron Energy Dependence of Fission Fragment Mass & TKE Distributions of 235U(n,f)- Reaction Below 10 eV, Dynamical Aspects of Nuclear Fission: Proceedings of the 4-th International Conference - Casta-Papiernicka: Slovak Republic, - ed. J. Kliman - World Scientific, Singapore, 2000. -P. 417-423.

- Zeynalov Sh., Hambsch F-J., Varapai N., Oberstedt S., Serot O., Prompt fission neutron emission in resonance fission of 239Pu,  XII International Seminar on Interaction of Neutrons with Nuclear – Dubna: JINR, -2004. –E3-2004-169. –P. 371-379.

- E.A.Sokol, Sh.S.Zeinalov, S. Šaro, M.Hussonnais, H.Brucherseifer, G.V.Bouklanov, M.P.Ivanov, Yu.S.Korotkin, V.I.Smirnov, L.P.Chelnokov, G.M.Ter-Akopian, G.N.Flerov, Preliminary results on the study of 259Md spontaneous fission parameters, JINR Rapid Communications  - 1986. – No. 19-86. P. 45-49.

- E.A.Sokol, Sh.S.Zeinalov, G.M.Ter-Akopian, Prompt fission neutron multiplicity of 256Fm, Atomnaya energiya – 1989. – Vol. 67, P. 357-358.

- N. Bohr, J.A. Wheeler, The mechanism of nuclear fission, Physical Review, 1939. - Vol. 56, No. 5. - P. 426-450.

- U. Brosa, S. Grossmann, Nuclear scission, A. Mũller, Physics Report, 1990. - Vol. 197, No. 4. - P. 167-262.

- Sh. Zeynalov, P. Sedyshev, V. Shvetsov, O. Sidorova, Prompt fission neutron investigation in 235U(nth,f) reaction, EPJ Web of Conferences 146, 04022 (2017) , DOI: 10.1051/epjconf/201714604022

- Sh. Zeynalov, P. Sedyshev, V. Shvetsov, O. Sidorova, Position sensitive twin ionization chamber for nuclear fission investigations, Applications of Nuclear Techniques (CRETE17), International Journal of Modern Physics: Conference Series, Vol. 48 (2018) 1860123 DOI: 10.1142/S2010194518601230

- Sh. Zeynalov, P. Sedyshev, V. Shvetsov, O. Sidorova, Prompt Fission Neutron Investigation in 235U(nth,f) and 252Cf(sf) Reactions, Wonder-2019, EPJ Web of Conferences 211, 04003 (2019),  DOI:10.1051/epjconf /2019/21104003

- Sh. Zeynalov, P. Sedyshev, O. Sidorova, V. Shvetsov, Nuclear Fission Investigation with Twin Ionization Chamber, Applications of Nuclear Techniques (CRETE19), International Journal of Modern Physics: Conference Series, in print

- Alf Gook, Franz-Josef Hambsch, and Stephan Oberstedt. EPJ Web of Conferences 1 , 05001 (2016)

- O.V. Zeynalova, Sh.S. Zeynalov, F.-J. Hambsch, S. Oberstedt, Bulletin of Russian Academy of Science: Physics, 73, 506-514 (2009).

- Sh. Zeynalov, O. Zeynalova, F.-J. Hambsch, S. Oberstedt, A new approach to prompt fission neutron TO data treatment, Physics Procedia 31 ( 2012) 132 – 140

- O. Zeynalova, Sh. Zeynalov , F.-J. Hambsch and S. Oberstedt, “DSP Algorithms for Fission Fragment and Prompt Fission Neutron Spectroscopy in Application of Mathematics in Technical and Natural Sciences-2010, edited by M. D. Todorov and C. I. Christov, AIP Conference Proceedings 1301, American Institute of Physics, Melville, NY, 2010, pp. 430-439

- Sh. Zeynalov, O. Zeynalova, F.-J. Hambsch, P. Sedyshev, V. Shvetsov, Ionization chamber for prompt fission neutron investigation, Physics Procedia 59 (2014) 160 – 166

- Zeynalov, S., Hambsch, F.-J., Oberstedt, S., 2011. ND-2013, Jour. Korean Phys. Soc. 59, 1396

- Al-Adili, D. Tarrio, F.-J. Hambsch, A. Gook, K. Jansson, A. Solders, V. Rakopoulos, C. Gustavson, M. Lantz, A. Materrs, S. Oberstedt, A.V. Prokofiev, M. Viladi, M. Osterlund, and S. Pomp,  EPJ Web of Conferences 122, 01007 (2016)

- Sh. Zeynalov, P. Sedyshev, O. Sidorova, V. Shvetsov, Applications of Nuclear Techniques (CRETE17), International Journal of Modern Physics: Conference Series, Vol. 48 (2018) 1860123.

- A.Gook, F.-J. Hambsch, and M. Vidali, Prompt neutron multiplicity in correlation

with fragments from spontaneous fission of 252Cf.  Phys. Rev. C 90, 064611 (2014)

- F.-J. Hambsch, H.-H. Knitter, C. Budtz-Jorgensen, and J.P. Theobald, Fission mode fluctuation in the resonances of 235U(n,f),  Nuclear Physics A -1989. -Vol. 491. –P. 56 – 90.

- A. Gook, W. Geerts, F.-J. Hambsch, S. Oberstedt, M. Vidali, Sh. Zeynalov A position sensitive twin ionization chamber for fission fragment and prompt neutron correlation experiments, Nuclear Instruments and Methods in Physics Research A830 (2016) 366

- A. Gook, F.-J. Hambsch, S. Oberstedt, Prompt neutron emission and energy balancer in 235U(n,f), ND-2016, EPJ Web of Conferences 146, 04007 (2017)

- B. Laurent, P. Marini, G, Belier, T. Bonnet, A. Chatillon, J. Taieb, D. Etasse, M.Devlin, R. Haight, New prompt fission neutron spectra measurements in the 238U(n,f) reaction with dedicated setup at LANCE/WNR, ND-2016, EPJ Web of Conferences 146, 04014 (2017).

- V.F. Apalin, Yu. N. Gtitsuk, I.E. Kutikov, V.I. Lebedev, and L.A. Mikaelyan, Nucl. Phys. 55, 249 (1964)

- S.B. Kaufman, E.P. Steinberg, B.D. Wilkins, J. Unik, A.J. Gorsky and M.J. Fluss, Nucl. Instr. and Meth. 115 (1974) 47.

- C. Budtz-Jorgensen, H.-H. Knitter, Ch. Straede, F.-J. Hambsch and R. Vogdt, Nucl. Instruments and Meth. A258 (1987) 209-220

- D. C. Vu, A. M. Sukhovoj, L. V. Мitsyna, Sh.Zeinalov, N. Jovancevic, D.Knezevic, M.Krmar, and A.Dragic, Representation of Radiative Strength Functions within a Practical Model of Cascade Gamma Decay, PHYSICS OF ATOMIC NUCLEI Vol. 80 No. 2 (2017)

- R.E. Howe, T.W. Phillips, C.D. Bowman, Phys. Rev. C 13, 195 (1976)