Transfer Line

Nuclotron-to-Collider

Valeri Lebedev & Victor Smirnov

JINR

July 2024

## Rms Bunch Length and Momentum Spread Coming out from Nuclotron

Maximal longitudinal beam emittance is determined by longitudinal Booster acceptance at injection

Bunch length and momentum spread in Nuclotron in the course of acceleration

The maximum rms momentum spread is 4∙10-4 at lowest collider energy of 1 GeV

Accounting 3s and adding relative energy error of 8∙10-4 we obtain the total required momentum acceptance 2∙10-3

This momentum spread will be sufficient for operation with p

## Required Transverse Acceptance of Transfer Line

The transverse beam emittance was chosen so that 2.5s beam size would be at the boundary of electron beam (14 mm) determined by longitudinal Booster acceptance at injection.

  - That yields en=0.2 mm

      - e=0.11 mm at 1 GeV

We require acceptance of 5s

      - eaccept ≥ 3 mm

  - Plus 5 mm for the orbit distortion

Actual acceptance of the line for 1.5 mm thick round chamber is 12 mm

Rms beam emittance in Nuclotron in the course of acceleration

## Beta Functions and Dispersions Through the Line

Peak beta-function 66 m

Peak dispersions and momentum acceptances

  - Horizontal – 8.8 m (Dp/pmax=2.7 cm/8 m = 3.4∙10-3)

  - Vertical – 5 m (Dp/pmax=2.7 cm/5 m = 5.4∙10-3)

Separate powering of quads and dipoles gives

  - considerable flexibility in optics choice

  - enables quad centering

## Beam Sizes Through the Line

Beam boundaries for 1 GeV beam: e95%=0.66 mm∙mrad, Dp/p=10-3 (2.5s)

Round vacuum chamber with OD 58 mm and 2 mm wall looks as reasonable choice

It supports: 
Transverse acceptance of 12 mm mrad

- Momentum acceptance 3.4∙10-3

## Eddy Currents in the Vacuum Chamber walls

Eddy currents  and, in the first approximation () are equal to zero

For OD 56 mm and 2 mm vacuum chamber one obtains

Thus, the eddy currents do not represent a problem

## Requirements for Correctors

Since the dipoles have independent power supplies and can be used as correctors we need 10 single plane correctors